# Project Proposal CTAI - Energy

Client:

Jeroen De Baets - Energy Lab

jeroen.debaets@howest.be

🎯 What

Households and small companies are consuming more and more energy due to the advent of electric cars, heat pumps and home batteries. Classic measurement methods to analyze that consumption are expensive and require complex hardware.

We want to investigate how AI can help to analyze energy consumption smarter and cheaper based on existing measurement data from InfluxDB.

Specifically, we want to develop a proof of concept that automatically learns which appliances are active based on a building's total energy measurement.

In other words, can AI infer from one overall energy measurement which appliances are currently consuming and how much each appliance contributes to the total?

🧩 Data

https://drive.google.com/drive/folders/1ZQnTWegUMxJ8y140FdLF28o3TpcaXQx9?usp=sharing

We have a dataset (JVR - 1 year) with consumption measurements by appliance and by measurement type.

The appliances are currently listed under each other in the data (hood, dryer, etc.).

Important fields

- msr_subject → name of the appliance (e.g. Dishwasher, Heat Pump, ...)

- msr_category → type of measurement (e.g. power_active, power_reactive, ...)

Important measurements

- Grid → total measurement of Fluvius = total consumption - production (solar panels)

- Building - consumption - Total - Smappee → total consumption of the building (measured)

- Building - consumption - Total - Calculated → total consumption of the building (calculated, should match Smappee)

- Building - production → production of solar panels

- Building - power_reactive → reactive power, not relevant for this case

💡 Goal

A solution that based on aggregated measurement data:

- Enables recognition of active devices.

  - The system learns to associate patterns of consumption with specific appliances.

- Predicts how much each appliance consumes, even without individual sensors.

- Identifies large consumers and detects creeping consumption.

In addition, we want to:

- Develop an interactive dashboard in which users:

  - consumption per appliance,

  - peak load,

  - and overall energy trends.

- Make recommendations on how to further optimize this project, including:

  - which AI techniques are most appropriate (e.g., sequence models, anomaly detection, deep learning),

  - how much and what type of data is needed to improve accuracy,

  - how future datasets can be better structured or labeled for training